# 1. Work included

CourtTech provides all material, labor, equipment and services necessary to install squash courts as specified and shown in the drawings. The squash courts are manufactured and distributed by

CourtTech GmbH & Co. KG,

Pullacher Str. 11,

83358 Seebruck,

Germany

Tel. +49 8667 72 – 490,

Fax. +49 8667 72 – 498,

info@courttech.biz.

www.courttech.biz

# 2. Approvals

All type of squash court walls, glass walls, floors, height adjustable tin have to be approved and certified by the World Squash Federation.

The supplier of the squash court components must have the Complete Court Accreditation (CCA) of the World Squash Federation. Only courts completed after 1st January 2013 with Complete Court Accredited will be considered as championship quality and thus eligible for hosting WSF World Championship and PSA / WSA Tour events.

# 3. Warranty

The structure and playing surfaces is guaranteed against defects under normal use for five years. This warranty period does not cover glass breakage or line markings.

# 4. Submittals

1. Detailed materials and fabrication specifications and installation instructions.

2. Installation and operating instructions.

3. Shop drawings for the erection and installation of the squash court components. The shop drawings indicate all builder's work requirements, dimensions of openings, all fixing details, suspension systems etc.

# 5. 7. Installation

1. Install courts complete, utilizing only qualified and approved personnel trained in correct installation techniques with experience on similar projects.

2. During Installation the temperature should range from 14° to 35° Celsius.

# 6. Components

6.1. System Front Wall

Squash, width: 6400 mm, height 4850 mm

Hardball Doubles: 7620 mm, height 6706 mm

Total thickness of the wall is 94mm, consisting of two 21mm thick RS special high density panels attached to steel c-profiles. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance squash court coating is applied to the wall surface, color RAL 9001. The system wall is filled with kiln-dried sand. The wall system is hard, flat and free of joints after installation.

# 6.2. System Side Wall

Squash, length: 9750 mm, height 4570 mm sloping down to 2130 mm

Hardball Doubles: 7620 mm, height 6096 with one step down to 4572 mm

Total thickness of the wall is 94mm, consisting of two 21mm thick RS special high density panels attached to steel C-profiles. Free standing wall sections are mounted onto steel plates fixed to the floor in three positions. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance coating is applied to the wall surface, color RAL 9001. The system wall is filled with 2½ tons of kiln-dried sand. The wall system is hard, flat and free of joints after installation. The out-of-play lines are made from extruded aluminum, color red RAL 3001.

# 6.3. Squash Movable System Side Wall

Total thickness of wall is 94mm, consisting of two 21mm thick RS special high density boards attached to steel C-profiles. The complete wall is suspended on two rails which are fixed over the glass back wall and the front wall. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance coating is applied to the wall surface, color RAL 9001. The movable system wall is filled with lighter filler than static walls. The wall is hard, flat and free of joints. The out-of-play lines are made from extruded aluminum, color red RAL 3001.

# 6.4. Squash Half System Front Wall

Consists of 21mm thick RS high density board fixed onto existing walls of the building with special concealed fixing brackets. The Half System Front Wall has a total construction thickness of 40mm. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance coating is applied to the wall surface, color RAL 9001. The gap between the building wall and the Half System wall is filled with minimum 1 ton of kiln-dried sand. The wall surface is flat and free of joints after installation

# 6.5. Squash Half System Side Wall

Consists of 21mm thick RS high density board fixed onto existing walls of the building with special concealed fixing brackets. The Half System Front Wall has a total construction thickness of 40mm. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance coating is applied to the wall surface, color RAL 9001.The wall surface is flat and free of vertical joints. The Half System Side Wall is filled with minimum 1 ton of kiln-dried sand. The out-of-play lines of the side wall are made from extruded aluminum, color red, RAL 3001.

6.6. Hardball Doubles System Front Wall

Total thickness of the wall is 94mm, consisting of two 21mm thick RS special high density panels attached to steel c-profiles. The walls are painted with three factory applied coats of high performance paint. On completion a final finish of high performance squash court coating is applied to the wall surface, color RAL 9001. The system wall is filled with kiln-dried sand. The wall system is hard, flat and free of joints after installation.

6.6. Glass side or front wall

Consists of several equal size panels of 12 mm thick toughened one-pane safety glass with ceramic double dots burnt into the glass. 50% of the glass is covered with these dots. The dot facing the outside of the court is black and the dot facing the inside of the court is colored in blue or green. The glass panels are supported by glass fins 400 mm long in a 90 degree angle.

# 6.7. Height adjustable Tin

This tin is height adjustable from the standard height of 480mm to 430mm (measured from finished floor level). The tin is made of 21mm thick high density boards, comprising two panels each 3195mm long and painted with two coats of high performance coating. The top profile of the tin is made of extruded aluminum, color red, RAL 3001.

# 6.7.1. Tin (alternative option to height adjustable tin)

As model 6.7 above this tin is made of 21mm thick high density boards, comprising three panels each 2130 mm long and painted with two coats of high performance coating. The top profile is made of extruded aluminum, color red, RAL 3001. The height of the tin is 480 mm.

# 6.8. Nets over front, side or back wall

Nets are made of 3mm thick polypropylene string, width of mesh approx. 20 mm, UV-proof. Nets are attached with hooks every 100mm and stretched with a 6mm steel cable at a height of 6000mm (height can be modified).

# 6.9. Head pieces above front, side or back wall

Boards made of 21mm thick RS high density board with two coats of high performance coating. The head pieces are a standard height of 6000mm from finished floor level (height can be modified).

# 6.10. Frame glass back wall

Clear width for Squash: 6400 mm

Clear width for Hardball Doubles: 7620 mm

The glass back wall consists of two panels and one glass door. The toughened 12 mm thick, one-pane safety glass with etching, has 2mm wide white ceramic silk-screened lines bonded into the glass up to a height ot 610mm. The clear entrance width is 914mm (suitable for wheelchair access). The glass panels are supported by an aluminum frame. The glass door is mounted laterally with cam hinges and door opens to the inside of the court.

# 6.10.1. Fin glass back wall (alternative option to frame glass back wall)

The glass wall consists of two or four panels and one glass door. The toughened 12 mm thick one-pane safety glass with etching has 2mm wide white ceramic silk-screened lines bonded into the glass up to a height ot 610mm. The clear entrance width is 914mm (suitable for wheelchair access). The glass panels are supported by glass fins 300mm wide and 2130mm high, fixed in at right angles to the glass panels. The glass door is mounted laterally with hinges and opens to the inside of the court.

# 6.11. Junckers SylvaSquash Sport Floor

The Junckers SylvaSquash sport floor is consists of 22mm solid beech tongued and grooved floor-boards secret-nailed to a resilient undercarriage system of semi-sprung. The floor system is an area elastic type of sports floor with high shock absorbency and elasticity suitable in multi-purpose sports hall as well as for squash. The construction height is 74mm without packing. The sports floor system fully conforms to EN 14904:A4

# 6.12. Squash LED Lighting

The court is illuminated by ten units each consisting of 2 LED tubes. The level of illumination measured one meter above finished floor level should be 500 lux. The walls of the court should be lit in such a way as to appear evenly and uniformly illuminated and lux levels should not vary at any point by more than 15% from the average level of illumination.